#include "lps22.h"

namespace esphome {

namespace lps22 {

static constexpr const char *const TAG = "lps22";

static constexpr uint8_t WHO_AM_I = 0x0F;

static constexpr uint8_t LPS22HB_ID = 0xB1;

static constexpr uint8_t LPS22HH_ID = 0xB3;

static constexpr uint8_t LPS28DFW_ID = 0xB4;

static constexpr uint8_t CTRL_REG2 = 0x11;

static constexpr uint8_t CTRL_REG2_ONE_SHOT_MASK = 0b1;

static constexpr uint8_t STATUS = 0x27;

static constexpr uint8_t STATUS_T_DA_MASK = 0b10;

static constexpr uint8_t STATUS_P_DA_MASK = 0b01;

static constexpr uint8_t TEMP_L = 0x2b;

static constexpr uint8_t PRES_OUT_XL = 0x28;

static constexpr uint8_t REF_P_XL = 0x28;

static constexpr uint8_t READ_ATTEMPTS = 10;

static constexpr uint8_t READ_INTERVAL = 5;

static constexpr float PRESSURE_SCALE = 1.0f / 4096.0f;

static constexpr float TEMPERATURE_SCALE = 0.01f;

void LPS22Component::setup() {

uint8_t value = 0x00;

this->read_register(WHO_AM_I, &value, 1);

if (value != LPS22HB_ID && value != LPS22HH_ID && value != LPS28DFW_ID) {

ESP_LOGW(TAG, "device IDs as %02x, which isn't a known LPS22HB or LPS22HH ID", value);

this->mark_failed();

}

}

void LPS22Component::dump_config() {

ESP_LOGCONFIG(TAG, "LPS22:");

LOG_SENSOR("  ", "Temperature", this->temperature_sensor_);

LOG_SENSOR("  ", "Pressure", this->pressure_sensor_);

LOG_I2C_DEVICE(this);

LOG_UPDATE_INTERVAL(this);

}

void LPS22Component::update() {

uint8_t value = 0x00;

this->read_register(CTRL_REG2, &value, 1);

value |= CTRL_REG2_ONE_SHOT_MASK;

this->write_register(CTRL_REG2, &value, 1);

this->set_retry(READ_INTERVAL, READ_ATTEMPTS, [this](uint8_t _) { return this->try_read_(); });

}

RetryResult LPS22Component::try_read_() {

uint8_t value = 0x00;

this->read_register(STATUS, &value, 1);

const uint8_t expected_status_mask = STATUS_T_DA_MASK | STATUS_P_DA_MASK;

if ((value & expected_status_mask) != expected_status_mask) {

ESP_LOGD(TAG, "STATUS not ready: %x", value);

return RetryResult::RETRY;

}

if (this->temperature_sensor_ != nullptr) {

uint8_t t_buf[2]{0};

this->read_register(TEMP_L, t_buf, 2);

int16_t encoded = static_cast<int16_t>(encode_uint16(t_buf[1], t_buf[0]));

float temp = TEMPERATURE_SCALE * static_cast<float>(encoded);

this->temperature_sensor_->publish_state(temp);

}

if (this->pressure_sensor_ != nullptr) {

uint8_t p_buf[3]{0};

this->read_register(PRES_OUT_XL, p_buf, 3);

uint32_t p_lsb = encode_uint24(p_buf[2], p_buf[1], p_buf[0]);

this->pressure_sensor_->publish_state(PRESSURE_SCALE * static_cast<float>(p_lsb));

}

return RetryResult::DONE;

}

}  // namespace lps22

}  // namespace esphome